AsmL Specification of UTOPIA Transactor with Memory Access

Tareq Hasan Khan

Declaration of Hardware Data types and Constants

namespace nsHardwareDatatype

// This namespace must be included in every specification

import System

// Constants

public const LOGIC_0 = '0'

public const LOGIC_1 = '1'

public const LOGIC_X = 'X'

public const LOGIC_Z = 'Z'

// Datatypes

public type Logic = Char

public type Lv_2 = String where (Size (value) <= 2)

public type Lv_4 = String where (Size (value) <= 4)

public type Lv_8 = String where (Size (value) <= 8)

public type Lv_16 = String where (Size (value) <= 16)

public type Lv_32 = String where (Size (value) <= 32)

public type Lv_64 = String where (Size (value) <= 64)

// LogicVector to Integer Convertion Function

public toInt ( s as String ) as Integer

var i as Integer = 0

var n as Integer = 0

var l as Integer = Size(s)

step while i <> l

require (ToInteger(s(i)) = 48 or ToInteger(s(i)) = 49)

n := n + (ToInteger (s(i)) - 48) * ToInteger(Math.Pow (2, (l - i - 1 )))

i := i + 1

step

return (n)

// Integer to LogicVector Convertion Function

public toLv ( n as Integer ) as String

var q as Integer = 0

var r as Integer = 0

var m as Integer = 0

var cstate as Integer = 1

var s as String = ""

var sr as String = ""

var i as Integer = 0

var l as Integer = 0

step

m := n

step while cstate <> 3

if cstate = 1 then

q := ToInteger ( m / 2 )

r := m mod 2

cstate := 2

if cstate = 2 then

s := s + ToString (r)

if q > 0 then

m := q

cstate := 1

else

cstate := 3

step

l := Size (s)

i := 0

step while i <> l

sr := sr + s( l - i - 1 )

i := i + 1

step

return (sr)

// Set Clock Signal Function

public SetClockSignal ( clk as Logic, isPosEdge as Boolean ) as Boolean

return true

Declaration of RTL Ports

namespace nsRTL

import nsHardwareDatatype

// All the RTL signals are declared here

public var TxData as Lv_8 = "XXXXXXXX"

public var TxSOC as Logic = LOGIC_0

public var TxEn_ as Logic = LOGIC_1

public var TxClav as Logic = LOGIC_0

public var TxClk as Logic = LOGIC_1

public var RxData as Lv_8 = "00000000"

public var RxSOC as Logic = LOGIC_0

public var RxEn_ as Logic = LOGIC_1

public var RxClav as Logic = LOGIC_0

public var RxClk as Logic = LOGIC_1

TLM Function Name: SendCell ()

namespace nsSendCell

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

// Function Declaration

public SendCell (  StartCellNo as Integer, EndCellNo as Integer, SrcCell as Seq of Integer )

// Declare Constants here

// Declare Local AsmL Variables here

// Specify the RTL Clock Signal here

// Start writing State Machine from here

TLM Function Name: GetCell ()

namespace nsGetCell

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

// Function Declaration

public GetCell (  DstCell as Seq of Integer )

// Declare Constants here

// Declare Local AsmL Variables here

// Specify the RTL Clock Signal here

// Start writing State Machine from here

TLM Function Name: nb_SendCell ()

namespace nsnb_SendCell

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

// Function Declaration

public nb_SendCell (  StartCellNo as Integer, EndCellNo as Integer, SrcCell as Seq of Integer, isCompleted as Boolean )

// Declare Constants here

// Declare Local AsmL Variables here

// Specify the RTL Clock Signal here

// Start writing State Machine from here

TLM Function Name: nb_GetCell ()

namespace nsnb_GetCell

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

// Function Declaration

public nb_GetCell (  DstCell as Seq of Integer, isCompleted as Boolean )

// Declare Constants here

// Declare Local AsmL Variables here

// Specify the RTL Clock Signal here

// Start writing State Machine from here

Call TLM Functions for Execution and Debugging

namespace nsMain

import nsHardwareDatatype

import nsRTL

import nsSendCell

import nsGetCell

import nsnb_SendCell

import nsnb_GetCell

Main()

step

WriteLine ( "UTOPIA Transactor with Memory Access" )