AsmL Specification of SystemC TLM-RTL Transactor

Tareq Hasan Khan

Declaration of Hardware Data types and Constants

namespace nsHardwareDatatype

// This namespace must be included in every specification

import System

// Constants

public const LOGIC_0 = '0'

public const LOGIC_1 = '1'

public const LOGIC_X = 'X'

public const LOGIC_Z = 'Z'

// Datatypes

public type Logic = Char

public type Lv_2 = String where (Size (value) <= 2)

public type Lv_4 = String where (Size (value) <= 4)

public type Lv_8 = String where (Size (value) <= 8)

public type Lv_16 = String where (Size (value) <= 16)

public type Lv_32 = String where (Size (value) <= 32)

public type Lv_64 = String where (Size (value) <= 64)

// LogicVector to Integer Convertion Function

public toInt ( s as String ) as Integer

var i as Integer = 0

var n as Integer = 0

var l as Integer = Size(s)

step while i <> l

require (ToInteger(s(i)) = 48 or ToInteger(s(i)) = 49)

n := n + (ToInteger (s(i)) - 48) * ToInteger(Math.Pow (2, (l - i - 1 )))

i := i + 1

step

return (n)

// Integer to LogicVector Convertion Function

public toLv ( n as Integer ) as String

var q as Integer = 0

var r as Integer = 0

var m as Integer = 0

var cstate as Integer = 1

var s as String = ""

var sr as String = ""

var i as Integer = 0

var l as Integer = 0

step

m := n

step while cstate <> 3

if cstate = 1 then

q := ToInteger ( m / 2 )

r := m mod 2

cstate := 2

if cstate = 2 then

s := s + ToString (r)

if q > 0 then

m := q

cstate := 1

else

cstate := 3

step

l := Size (s)

i := 0

step while i <> l

sr := sr + s( l - i - 1 )

i := i + 1

step

return (sr)

// Set Clock Signal Function

public SetClockSignal ( clk as Logic, isPosEdge as Boolean ) as Boolean

return true

Declaration of RTL Ports

namespace nsRTL

import nsHardwareDatatype

// All the RTL signals are declared here

public var AddressBus as Lv_32 = "00000000000000000000000000000000"

public var WR_DataBus as Lv_32 = "00000000000000000000000000000000"

public var RD_DataBus as Lv_32 = "00000000000000000000000000000000"

public var r_wb as Logic = LOGIC_0

public var Enable as Logic = LOGIC_0

public var Ack as Logic = LOGIC_0

public var Err as Logic = LOGIC_0

public var clk as Logic = LOGIC_1

TLM Function Name: mem_write ()

namespace nsmem_write

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

enum typeState

S_OpenWriteWindow

S_Ack

S_CloseWriteWindow

S_End

// Function Declaration

public mem_write ( adr as Integer, data as Integer, err as Boolean )

// Declare Constants here

// Declare Local AsmL Variables here

var C_State as typeState = S_OpenWriteWindow

// Specify the RTL Clock Signal here

SetClockSignal ( clk , true )

// Start writing State Machine from here

step while ( C_State <> S_End )

match ( C_State )

S_OpenWriteWindow :

// write address

WriteLine ( "Write Address" )

AddressBus := toLv ( adr )

// write data

WriteLine ( "Write Data")

WR_DataBus := toLv ( data )

// r_wb = 0

WriteLine ("r_wb = 0")

r_wb := LOGIC_0

// enable = 1

WriteLine ("Enable = 1")

Enable := LOGIC_1

// next state

C_State := S_Ack

S_Ack :

WriteLine ("Ack (1/0)? : ")

Ack = ToChar (ReadLine ())

// wait for ack

if Ack = LOGIC_1 then

C_State := S_CloseWriteWindow

else

C_State := S_Ack

S_CloseWriteWindow :

// write enable = 0

WriteLine ("Enable = 0")

Enable := LOGIC_0

WriteLine ("Err (1/0)? : ")

Err = ToChar ( ReadLine() )

// read err

if Err = LOGIC_1 then

// err := true

WriteLine ( "err = 1" )

else

// err := false

WriteLine ( "err = 0" )

// next state

C_State := S_End

TLM Function Name: mem_read ()

namespace nsmem_read

import nsHardwareDatatype

import nsRTL

// Declare Enumeration here

enum typeState

S_OpenReadWindow

S_Ack

S_CloseReadWindow

S_End

// Function Declaration

public mem_read ( adr as Integer, data as Integer, err as Boolean )

// Declare Constants here

// Declare Local AsmL Variables here

var C_State as typeState = S_OpenReadWindow

// Specify the RTL Clock Signal here

SetClockSignal ( clk , true )

// Start writing State Machine from here

step while ( C_State <> S_End )

match ( C_State )

S_OpenReadWindow :

// write address

WriteLine ( "Write Address" )

AddressBus := toLv ( adr )

// r_wb = 1

WriteLine ( "r_wb = 1" )

r_wb := LOGIC_1

// enable = 1

WriteLine ( "Enable = 1" )

Enable := LOGIC_1

// next state

C_State := S_Ack

S_Ack :

WriteLine ( "Ack (1/0)? : " )

Ack = ToChar (ReadLine())

// wait for ack

if Ack = LOGIC_1 then

C_State := S_CloseReadWindow

else

C_State := S_Ack

S_CloseReadWindow :

// write enable = 0

WriteLine ( "Enable = 0" )

Enable := LOGIC_0

// read data

WriteLine ( "RD_DataBus ? : " )

RD_DataBus = ReadLine ()

// data := toInt ( RD_DataBus )

WriteLine ( "data = " + toInt (RD_DataBus) )

WriteLine ( "Err (1/0) ? : ")

Err = ToChar (ReadLine())

// read err

if Err = LOGIC_1 then

// err := 1

WriteLine ( "err = 1" )

else

// err := 0

WriteLine ( "err = 0" )

// next state

C_State := S_End

Call TLM Functions for Execution and Debugging

namespace nsMain

import nsHardwareDatatype

import nsRTL

import nsmem_write

import nsmem_read

var radr as Integer = 1

var rdata as Integer = 0

var rerr as Boolean = false

var wadr as Integer = 1

var wdata as Integer = 50

var werr as Boolean = false

Main()

step

WriteLine ( "TLM-RTL Transactor for Memory" )

step

WriteLine ("Calling mem_write () function...")

mem_write ( wadr , wdata , werr )

step

WriteLine ("Calling mem_read () function...")

mem_read ( radr , rdata , rerr )